| Q No | Question and options | Answer |
| --- | --- | --- |
| Q1 | Characteristics of an ideal paint include: - Paint should show hair cracks on drying - Paint should give attractive appearance - It should form film of uniform colour on drying - It should form a hard and durable surface | II, III and IV |
| A | I, II and III |  |
| B | II, III and IV |  |
| C | II and III |  |
| D | I, II, III and IV |  |
| Q2 | Example of a drier used in varnish is: | Litharge |
| A | Litharge |  |
| B | Copal |  |
| C | Turpentine |  |
| D | Mastic |  |
| Q3 | Heating asphalt with sand and mineral fillers produces: | Mastic Asphalt |
| A | Mastic Asphalt |  |
| B | Asphaltic felt |  |
| C | Asphaltic terrazzo |  |
| D | Distilled asphalt |  |
| Q4 | In industrial buildings, to obtain a hard-wearing surface, the finishing provided above concrete topping is: | Granolithic finishing |
| A | Granolithic finishing |  |
| B | Basaltic finishing |  |
| C | Marble finishing |  |
| D | Sand finish |  |
| Q5 | Type of stone used generally for masonry work in industrial area exposed to smoke and chemical fumes is: | Granite |
| A | Granite |  |
| B | Limestone |  |
| C | Sandstone |  |
| D | Marble |  |
| Q6 | Timber flooring is susceptible to: | All of the above |
| A | Termite attack |  |
| B | Dampness |  |
| C | Warping |  |
| D | All of the above |  |
| Q7 | The time elapsed between the moment at which water added to the Ordinary Portland cement and the moment at which Ordinary Portland cement paste completely lost its plasticity, should not be greater than | 600minutes |
| A | 500minutes |  |
| B | 600minutes |  |
| C | 700minutes |  |
| D | 800minutes |  |
| Q8 | When Unaerated ordinary rapid hardening cement tested by ‘Le Chatelier’ method as per IS: 4031 – 1988, it shall not have an expansion of more than: | 10mm |
| A | 10mm |  |
| B | 20mm |  |
| C | 25mm |  |
| D | 30mm |  |
| Q9 | Initial setting time (in minutes) for low heat cement when tested by Vicat’s apparatus as per IS: 12600 – 1989 is: | 60 |
| A | 30 |  |
| B | 60 |  |
| C | 90 |  |
| D | 120 |  |
| Q10 | When rapid hardening cement tested for fineness in terms of specific surface by Blaine’s air permeability test as per IS: 4031 – 1988, the specific surface must not be less than: | 325 m2/kg |
| A | 225 m2/kg |  |
| B | 325 m2/kg |  |
| C | 375 m2/kg |  |
| D | 400 m2/kg |  |
| Q11 | The Pigment Volume Concentration Number (PVCN) is given by, Where V1 – Volume of pigment in the paint and V2 – Volume of non-volatile or carrier in the paint. | V1/(V1+V2) |
| A | V1V2/(V1+V2) |  |
| B | V1/(V1+V2) |  |
| C | V2/(V1+V2) |  |
| D | V1/V2 |  |
| Q12 | Asbestos cement sheets possess which of the following properties? - Fire resistant - Vermin resistant - Water resistant | I, II and III |
| A | I and II |  |
| B | I and III |  |
| C | II and III |  |
| D | I, II and III |  |
| Q13 | Hard, lustrous resin obtained from ground where pine trees existed in the past is: | Copal |
| A | Copal |  |
| B | Lac |  |
| C | Amber |  |
| D | Mastic |  |
| Q14 | When tested as per IS: 4031 – 1988, the heat of hydration of low heat Portland cement when tested for 7 days shall not be more than: | 65 cal/g |
| A | 55 cal/g |  |
| B | 65 cal/g |  |
| C | 75 cal/g |  |
| D | 85 cal/g |  |
| Q15 | Short horizontal pieces used in stud partitions made of timber are called: | Noggins |
| A | Noggins |  |
| B | Studs |  |
| C | Braces |  |
| D | Clamp |  |
| Q16 | Bituminous slates used for roofing, generally in hilly areas, are: | Eternits |
| A | mastic slates |  |
| B | Trafford slates |  |
| C | Eternits |  |
| D | Slates |  |
| Q17 | Burnt umber provides a tint of which colour | Brown |
| A | Black |  |
| B | Blue |  |
| C | Brown |  |
| D | Red |  |
| Q18 | The PVCN range for prime coat on metals is: | 25-40 |
| A | 25-40 |  |
| B | 35-40 |  |
| C | 55-70 |  |
| D | 60-85 |  |
| Q19 | The cement which is same as blast furnace cement except that the blast furnace slag is ground wet and separately from cement is: | Trief cement |
| A | Trief cement |  |
| B | Oil well cement |  |
| C | Expansive cement |  |
| D | High alumina cement |  |
| Q20 | Varnishes which are methylated spirit of wine in which soft resins are dissolved are known as: | Lacquers |
| A | Oil varnishes |  |
| B | Lacquers |  |
| C | Turpentine varnishes |  |
| D | Cutback |  |
| Q21 | Short horizontal pieces used in stud partitions made of timber are called: | Noggins |
| A | Noggins |  |
| B | Studs |  |
| C | Braces |  |
| D | Clamp |  |
| Q22 | Timber lintels strengthened by the provision of mild steel plates at their top and bottom are called: | Flitched lintels |
| A | Flitched lintels |  |
| B | Trief lintels |  |
| C | Lacquer lintels |  |
| D | Matic linintels |  |
| Q23 | A road embankment 12m wide at formation level with side slope of 1:1 and average depth of 5m is constructed with an average gradient of 1 in 50 from contours 200m to 300m. Find the volume of earthwork. | 425000 m3 |
| A | 125000m3 |  |
| B | 225000 m3 |  |
| C | 325000 m3 |  |
| D | 425000 m3 |  |
| Q24 | ‘Value of a building’ indicates | Present value (Rs) of a building |
| A | Original cost of construction |  |
| B | Present value (Rs) of a building |  |
| C | Loan taken for the construction of building |  |
| D | Annual repair cost |  |
| Q25 | Which of the following is not a purpose of valuation | Project planning |
| A | Tax calculation |  |
| B | Rent fixation |  |
| C | Buying and selling property |  |
| D | Project planning |  |
| Q26 | A power generator is installed in an apartment at a cost of Rs 250000. Assuming the life of the generator as 15years, calculate the annual installment of sinking fund. interest =4% compounded annually | Rs 12500 |
| A | Rs 12500 |  |
| B | Rs 25000 |  |
| C | Rs 10000 |  |
| D | Rs 12000 |  |
| Q27 | A person constructed a warehouse at a cost of Rs 4,00,000/- excluding the cost of land. The warehouse becomes obsolete after 10 years. Find the amount of sinking fund to be annually deposited at the rate of 5% compound interest. The scrap value of the warehouse is 10% of the cost of purchase. | Rs 28622 |
| A | Rs 31800 |  |
| B | Rs 28622 |  |
| C | Rs 26622 |  |
| D | Rs 31600 |  |
| Q28 | Determine the amount of yearly depreciation for the property purchased at a cost of Rs. 20,00,000/-and having useful life of 20 years. | Rs 1,00,000 per year |
| A | Rs 90,000 per year |  |
| B | Rs 60,000 per year |  |
| C | Rs 1,00,000 per year |  |
| D | Rs 95,000 per year |  |
| Q29 | A first class building with built up area 300m2 was purchased at the rate of Rs 350/- per square meter. Determine the value of property after 30years if the life is 100years.take fixed percentage of depreciation as 1. | Rs 77668 |
| A | Rs 77668 |  |
| B | Rs 73500 |  |
| C | Rs 71000 |  |
| D | Rs 60000 |  |
| Q30 | The following is not considered in unit rate analysis of an item of building. | Contingency expenses |
| A | Present rate of materials |  |
| B | Contractor’s profit |  |
| C | Contingency expenses |  |
| D | Present rate of skilled labour |  |
| Q31 | Which of the following building quantity are measured in cubic meter. | Brick masonry |
| A | Plastering |  |
| B | Damp proof course |  |
| C | Brick masonry |  |
| D | flooring |  |
| Q32 | Estimate the quantity of concrete for 15cm thick RCC slab over a room of dimension 3mx4m, 30 cm wall thick, and 0.25 m projection all round. | 2.3625 m3 |
| A | 2.3625 m3 |  |
| B | 7.875 m3 |  |
| C | 2.9147 m3 |  |
| D | 2.8917 m3 |  |
| Q33 | Which of the following is not an outgoing of a building | Scrap value of building |
| A | Scrap value of building |  |
| B | Tax on building |  |
| C | Repair of building |  |
| D | Electric charges for running the common facilities in building |  |
| Q34 | Deduction for plastering for opening having area greater than 0.5 sqm and less than 3 sq m is | made for one side of the wall and the jamps, soffit and sills are not taken into account. |
| A | made for Both the sides of the wall and the jamps, soffit and sills are taken into account and added. |  |
| B | Not made |  |
| C | made for one side of the wall and the jamps, soffit and sills are taken into account and added. |  |
| D | made for one side of the wall and the jamps, soffit and sills are not taken into account. |  |
| Q35 | Revised estimate is prepared is prepared when the original sanctioned detailed estimate exceeds by | 5% |
| A | 15% |  |
| B | 10% |  |
| C | 5% |  |
| D | 3% |  |
| Q36 | The booklet prepared by the P.W.D. or government containing detailed description of all items of work together with their current rates is known as | Schedule of rates |
| A | Analysis of rates |  |
| B | Costing |  |
| C | rating |  |
| D | Schedule of rates |  |
| Q37 | Deduction for openings in measurement of plastering is not done if the area of the opening is ________ | Less than 0.5 sq m |
| A | Less than 0.1sq.m |  |
| B | Less than 0.5 sq m |  |
| C | Less than 0.6 sq m |  |
| D | Less than 0.7 sq m |  |
| Q38 | Deduction for openings in measurement of plastering is done for both the faces if the area of the opening is ________ | Greater than 3sq. m |
| A | Greater than 3sq. m |  |
| B | Greater than 5sq. m |  |
| C | Greater than 2sq. m |  |
| D | Greater than 1sq. m |  |
| Q39 | Semi circle 2.1m 1.5m Calculate the deduction for door opening shown in figure to calculate quantity of 30cm brick masonry in superstructure. | 1.21 m3 |
| A | 3.17 m3 |  |
| B | 3.41 m3 |  |
| C | 1.21 m3 |  |
| D | 4.03 m3 |  |
| Q40 | Eatimate the quantity of steel for the inclined ties (ISA 50X50x6MM @ 4.5 Kg/m) in the struss given below. | 21.6 kg |
| A | 10.8kg |  |
| B | 32.4 kg |  |
| C | 16.2 kg |  |
| D | 21.6 kg |  |
| Q41 | Estimate the quantity of wood for frame (choukhat- 8cm x12cm) in window of size 1.5m x 1m, with 12cm horns. | 0.052m3 |
| A | 5.200m3 |  |
| B | 0.048m3 |  |
| C | 0.052m3 |  |
| D | 0.050m3 |  |
| Q42 | The trapezoidal cross section of road embankment in banking is having a formation width of 10m, side slopes of 2:1 (H:V) and average depth of 1.3m. Calculate the quantity of earthwork for 200m length. Assume that there is no transverse slope. | 3276 m3 |
| A | 3276 m3 |  |
| B | 3312 m3 |  |
| C | 3288 m3 |  |
| D | 3200 m3 |  |
| Q43 | To estimate the quantities of different items, determine the number of junctions for the given plan. ( wall thickness of all the walls is 30cm) | 6 |
| A | 5 |  |
| B | 6 |  |
| C | 4 |  |
| D | 7 |  |
| Q44 | Estimate the cost of colour washing of ceiling @ Rs2/- per sqm for the rooms and verandah in the given plan. . ( wall thickness of all the walls is 30cm) | 45.049 m2 |
| A | 45.049 m2 |  |
| B | 43.969m2 |  |
| C | 50.138m2 |  |
| D | 38.880m2 |  |
| Q45 | The value of a property at a particular year obtained by original cost minus amount of depreciation up to previous year is known as |  |
| A | Capitalized value |  |
| B | Market value |  |
| C | Selling price |  |
| D | Book value |  |
| Q46 | In estimating the cost for earthwork excavation an extra rate is provided for every lead of _____ and lift of ______ | 30m, 1.5m |
| A | 30cm, 1.5cm |  |
| B | 45m, 2m |  |
| C | 30m, 3m |  |
| D | 30m, 1.5m |  |
| Q47 | The estimated cost for preparation of 4m wide sub grade (dressing to camber) of a new state highway for one kilometer length at the rate of Rs 50.00 per sq m. | Rs 2,00,000 |
| A | Rs 2,000 |  |
| B | Rs 20,000 |  |
| C | Rs 2,00,000 |  |
| D | Rs 20,00,000 |  |
| Q48 | Quantity of steel from bar bending schedule of a is calculated in __________ | Kilograms |
| A | Meters |  |
| B | No. of full length steel rods |  |
| C | Kilograms |  |
| D | grams |  |
| Q49 | In bar bending schedule which one of the following particulars is not required | Breadth |
| A | Length |  |
| B | Breadth |  |
| C | Weight |  |
| D | Number of bars |  |
| Q50 | In trapezoidal method to calculate the volume of earthwork required it is assumed that the area of cross section varies | Linearly |
| A | Parabolically |  |
| B | Spirally |  |
| C | Linearly |  |
| D | Uniformly |  |
| Q51 | For estimating the painting work for fully paneled door _________ is multiplied to area of wall opening of door. | 2.25 |
| A | 2.25 |  |
| B | 1 |  |
| C | 2 |  |
| D | 3 |  |
| Q52 | The plinth projection for the wall is calculated as | Half the Difference between breadth of plinth and breadth of wall |
| A | Difference between plinth area and carpet area |  |
| B | Difference between breadth of plinth and breadth of wall |  |
| C | Half the Difference between breadth of plinth and breadth of wall |  |
| D | Difference of breadth of damp proof course and breadth of plinth wall |  |
| Q53 | In estimation the quantities of the following are same | Plastering and white washing |
| A | Damp proof course and Plinth wall |  |
| B | Plastering and white washing |  |
| C | Wood work and painting on wood work |  |
| D | Painting for fully paneled door and partly paneled and partly glazed |  |
| Q54 | Which one of the following is not the characteristics of real fluid | Incompressibility |
| A | Compressibility |  |
| B | Incompressibility |  |
| C | Surface tension |  |
| D | Viscosity |  |
| Q55 | Kinematic viscosity of air at 250c is 1.7x10-5 m2/s, its kinematic viscosity at 750c is | 2.4x10-5 m2/s |
| A | 0.4x10-5 m2/s |  |
| B | 0.8x10-5 m2/s |  |
| C | 1.6x10-5 m2/s |  |
| D | 2.4x10-5 m2/s |  |
| Q56 | The pressure within the droplet of water of 0.5mm in diameter, the atmospheric air pressure of 1.01bar is (assume surface tension at the water droplet as 0.09 N/m) | 101720 Pa |
| A | 101720 Pa |  |
| B | 107720 Pa |  |
| C | 117270 Pa |  |
| D | 117720 Pa |  |
| Q57 | The device used to measure the pressure difference between two points in the same or different pipes is known as | Differential manometer |
| A | Piezometer |  |
| B | Single column manometer |  |
| C | U tube manometer |  |
| D | Differential manometer |  |
| Q58 | Which of the following device is used to measure the pressure of the fluid flowing through the pipes is 100bar above the atmosphere. | Bourdon Pressure gauge |
| A | U tube manometer |  |
| B | Differential manometer |  |
| C | Bourdon Pressure gauge |  |
| D | Diaphragm Pressure Gauge |  |
| Q59 | The centre of pressure (hc) for a immersed inclined surface is | hc = hg + Ig/hgAsin2θ |
| A | hc = hg + Ig/hgA |  |
| B | hc = hg + Ig/hgAsin2θ |  |
| C | hc = hg + Ig/hgA2sin2θ |  |
| D | hc = hg + Ig/hgA2sinθ |  |
| Q60 | A rectangular lamina of size 4m x 2m is held in water vertically for a depth of 3m, then the depth of centre of pressure is | 5.266m |
| A | 5.266m |  |
| B | 7.388m |  |
| C | 8.324m |  |
| D | 9.564m |  |
| Q61 | The horizontal component of pressure force in a curved surface submerged in a stationary liquid is equal to | The pressure force on a vertical projection of the surface |
| A | The product of the surface area and pressure at the centre of gravity |  |
| B | The pressure force on a horizontal projection of the surface |  |
| C | The pressure force on a vertical projection of the surface |  |
| D | Weight of the liquid contained between curved and plane surface |  |
| Q62 | The force exerted by a static fluid on a inclined plane immersed surface is equal to | ρgAhg |
| A | ρgAhg |  |
| B | ρg + Ahg |  |
| C | ρg - Ahg |  |
| D | ρg / Ahg |  |
| Q63 | The Bernoullis equation is applicable to | Steady flow |
| A | Compressible flow |  |
| B | Steady flow |  |
| C | Non uniform flow |  |
| D | Unsteady flow |  |
| Q64 | For a steady, ideal flow of an incompressible fluid, the total energy at any point of the fluid is constant. This is the statement of | Bernoullis equation |
| A | Eulers equation |  |
| B | Momentum equation |  |
| C | Bernoullis equation |  |
| D | Moment of momentum equation |  |
| Q65 | The water is flowing through a converging pipe of 20cm and 15cm diameters positioned inclined whose inlet and outlet are at 6m and 3m above the datum. If the pressure at the inlet is 29.43 N/cm2 and rate of flow is 40litres/s, then the intensity of pressure at the outlet of the pipe is | 32.19 N/cm2 |
| A | 29.43 N/cm2 |  |
| B | 49.64 N/cm2 |  |
| C | 32.19 N/cm2 |  |
| D | 46.34 N/cm2 |  |
| Q66 | The energy loss in case of venturimeter when compared to orifice meter is | Less |
| A | Same |  |
| B | Less |  |
| C | More |  |
| D | May be less or more |  |
| Q67 | A orifice of diameter 8cm is fitted in a pipe of 18cm diameter carrying oil of specific gravity 0.85 and specific gravity of mercury 13.6. The head causing the flow is measured with the help mercury manometer which is 36cm. What is the flow rate of the oil passing through the pipe if coefficient of discharge is 0.6. | 0.03166 m3/s |
| A | 0.03166 m3/s |  |
| B | 0.06233 m3/s |  |
| C | 0.08466 m3/s |  |
| D | 0.04333 m3/s |  |
| Q68 | The type of turbine in which the direction of water flow through runner is axial | Kaplan |
| A | Pelton |  |
| B | Kaplan |  |
| C | Francis |  |
| D | Fourneyron |  |
| Q69 | The reason for using curved vanes over flat vanes in a hydraulic turbine is due to | Higher hydraulic efficiency can be |
| A | Low maintenance cost |  |
| B | Higher hydraulic efficiency can be obtained |  |
| C | Convenience of machining |  |
| D | High velocity jets can be used |  |
| Q70 | The approximate number of blades for Pelton turbine given by Tydun is | Z = (15 + D/2d) |
| A | Z = (15 + D/2d) |  |
| B | Z = (15 + 2D/d) |  |
| C | Z = (20 + D/2d) |  |
| D | Z = (20 + 2D/d) |  |
| Q71 | The degree of reaction is zero for which of the following type of turbine | Pelton |
| A | Pelton |  |
| B | Francis |  |
| C | Kaplan |  |
| D | Thomson |  |
| Q72 | Absolute blade velocity of a Kaplan turbine having width to diameter ratio 0.7, head of 70m, speed 500 rpm, flow ratio 0.2 and diameter of hub is 10 m is | 523.3 m/s |
| A | 653.3 m/s |  |
| B | 345.6 m/s |  |
| C | 523.3 m/s |  |
| D | 239.8 m/s |  |
| Q73 | Efficiency of draft tube is defined as the | Ratio of actual regain of pressure head to velocity head at draft tube entrance |
| A | Ratio of relative regain of pressure head to relative velocity head at draft tube entrance |  |
| B | Product of relative regain of pressure head to relative velocity head at draft tube entrance |  |
| C | Ratio of actual regain of pressure head to velocity head at draft tube entrance |  |
| D | Product of actual regain of pressure head to velocity head at draft tube entrance |  |
| Q74 | The centre of pressure for a plane vertical surface lies at a depth of -------- the height of immersed surface | Two-third |
| A | One- third |  |
| B | Two-third |  |
| C | Three-fourth |  |
| D | Three-eighth |  |
| Q75 | The maximum hydraulic efficiency for an impulse turbine with exit angle φ is | ((1 + cos φ)/2) |
| A | (1 - cos φ/2) |  |
| B | (1/2 + cos φ) |  |
| C | ((1 + cos φ)/2) |  |
| D | ((1- cos φ) /2) |  |
| Q76 | As per IS : 1172-1963, water required per head per day for average domestic purposes, is | 135 liters |
| A | 69 liters |  |
| B | 78 liters |  |
| C | 120 liters |  |
| D | 135 liters |  |
| Q77 | In slow sand filters, the turbidity of raw water can be removed only up to : | 60mg/lit |
| A | 60mg/lit |  |
| B | 125mg/lit |  |
| C | 250mg/lit |  |
| D | 300mg/lit |  |
| Q78 | A water supply scheme has to be designed for a city of population 100,000 people. The Estimated average daily draft if the average consumption is 250 lpcd is. | 25Mld. |
| A | 15Mld. |  |
| B | 25Mld. |  |
| C | 35Mld. |  |
| D | 45Mld. |  |
| Q79 | Two primary settling basins are 26m in diameter and 2.1m in depth. If the discharge is 26000 Cu.m per day, and the detention time is 2.40 hours, calculate the volume of the tank. | 1300Cu.m |
| A | 1200Cu.m |  |
| B | 1250Cu.m |  |
| C | 1300Cu.m |  |
| D | 1350Cu.m |  |
| Q80 | Most commonly used pump for lifting water in water supply mains, is | Axial flow pump |
| A | Centrifugal pump |  |
| B | Rotary pump |  |
| C | Reciprocating pump |  |
| D | Axial flow pump |  |
| Q81 | Distribution mains of any water supply, is normally designed for its average daily requirement: | 225% |
| A | 100%. |  |
| B | 150%. |  |
| C | 200%. |  |
| D | 225%. |  |
| Q82 | The population growth curve is a : | S- shaped Curve |
| A | S- shaped Curve |  |
| B | Parabolic curve |  |
| C | Circular curve |  |
| D | Straight line |  |
| Q83 | Turbidity of raw water is a measure of: | Suspended solids |
| A | Suspended solids |  |
| B | Acidity of water |  |
| C | BOD |  |
| D | COD |  |
| Q84 | The primary waste water treatment is aimed at: | Sedimentation |
| A | Filtration |  |
| B | Sedimentation |  |
| C | Coagulation |  |
| D | Settlement |  |
| Q85 | The self cleaning velocity of fine sand and clay is: | 0.15m/sec |
| A | 0.35m/sec |  |
| B | 0.25m/sec |  |
| C | 0.15m/sec |  |
| D | 0.05m/sec |  |
| Q86 | Calculate the velocity for a circular pipe of 1m diameter laid at a slope of 1 in 500, pipe running half full. Take rugosity factor of 0.012. | 1.48m/sec |
| A | 4.48m/sec |  |
| B | 3.48m/sec |  |
| C | 2.48m/sec |  |
| D | 1.48m/sec |  |
| Q87 | A stoneware sewer, 30 cm in diameter is laid at a gradient of 1in 100. Using N= 0.013, calculate the discharge when the sewer is running full. | 0.09 Cumec |
| A | 0.08 Cumec |  |
| B | 0.09 Cumec |  |
| C | 0.06 Cumec |  |
| D | 0.07 Cumec |  |
| Q88 | If 1% solution of a sewage sample is incubated for 5 days at 20°C and depletion of oxygen was found to be 3 ppm, B.O.D. of the sewage is | 300ppm |
| A | 200ppm |  |
| B | 225ppm |  |
| C | 250ppm |  |
| D | 300ppm |  |
| Q89 | During hydraulic design of sewers, if D is the diameter of upper circular portion, the overall depth of a standard egg shaped section, is | 1.50D |
| A | D |  |
| B | 1.25D |  |
| C | 1.50D |  |
| D | 2D |  |
| Q90 | If the diameter of sewer is 225 mm, the gradient required for generating self cleansing velocity is: | 1 in 120 |
| A | 1 in 60 |  |
| B | 1 in 90 |  |
| C | 1 in 120 |  |
| D | 1 in 160 |  |
| Q91 | Solid particles larger than colloidal particles are called: | Dust |
| A | Smoke |  |
| B | Dust |  |
| C | Fog |  |
| D | Aerosols |  |
| Q92 | The width of a rectangular sewer is twice its depth while discharging 1.5 m/sec. The width of the sewer is: | 1.36m |
| A | 0.68m |  |
| B | 0.88m |  |
| C | 1.36m |  |
| D | 1.76m |  |
| Q93 | For the survival of fish in a river stream, the minimum dissolved oxygen prescribed is: | 4ppm |
| A | 3ppm |  |
| B | 4ppm |  |
| C | 5ppm |  |
| D | 8ppm |  |
| Q94 | The coagulant widely used for sewage treatment, is: | Ferric chloride |
| A | Alum |  |
| B | Ferric chloride |  |
| C | Ferric Sulfate |  |
| D | Chlorine |  |
| Q95 | For evaporation and measurement of settlable solids, the apparatus used, is: | Imhoff cone |
| A | Jar |  |
| B | Beaker |  |
| C | Test tube |  |
| D | Imhoff cone |  |
| Q96 | Removal of oil and grease from sewage, is known as: | Skimming |
| A | Screening |  |
| B | Skimming |  |
| C | Filtration |  |
| D | Oxidation |  |
| Q97 | The sewage treatment units in which anaerobic decomposition of organic matter is used, are called: | Trickling filters |
| A | Imhoff tanks |  |
| B | Trickling filters |  |
| C | Sedimentation tanks |  |
| D | Digestors |  |
| Q98 | The digestion of settled sludge is carried out by: | Anaerobic decomposition |
| A | Aerobic decomposition |  |
| B | Anaerobic decomposition |  |
| C | Decomposition |  |
| D | Composition |  |
| Q99 | Settling the sewage in the presence of excess oxygen is called: | Activated Sludge |
| A | Activated Sludge |  |
| B | Deactivated Sludge |  |
| C | Digested Sludge |  |
| D | Indigested Sludge |  |
| Q100 | Heavy mineral material found in raw sewage is called: | Grit |
| A | Gravel |  |
| B | Grit |  |
| C | Glass |  |
| D | Silt |  |
| Q101 | Aerobic bacteria combining with oxygen is called: | Oxidation |
| A | Oxidation |  |
| B | Deoxidation |  |
| C | Decomposition |  |
| D | Reduction |  |
| Q102 | Determine the BOD for a sewage having 5-day BOD at 20°C as 160ppm with deoxygenating constant as 0.2 per day | 214ppm |
| A | 124ppm |  |
| B | 234ppm |  |
| C | 214ppm |  |
| D | 244ppm |  |
| Q103 | A sample of water has a 4 day BOD at 20°C as a value of 75% of the final. Find the rate constant to base 10 per day. | 0.15 per day |
| A | 0.45 per day |  |
| B | 0.35 per day |  |
| C | 0.15 per day |  |
| D | 0.25 per day |  |
| Q104 | Calculate the velocity through a rectangular sewer 2m x 1m deep laid to a slope of 1 in 200. Use Chezy’s Formula. Take Chezy’s constant as 67.65 | 27.62m/sec |
| A | 17.62m/sec |  |
| B | 27.62m/sec |  |
| C | 37.62m/sec |  |
| D | 47.62m/sec |  |
| Q105 | What is the minimum pitch distance according to IS 800:2007 if d is the bolt diameter? | 2.5d |
| A | 2d |  |
| B | d |  |
| C | 1.7d |  |
| D | 2.5d |  |
| Q106 | An ISMC 300 @ 0.363 kN/m is connected to a 12mm thick gusset plate. The size of the weld is 6 mm. Assume site welding. The strength of the weld is : | 663 N/mm |
| A | 600 N/mm |  |
| B | 750 N/mm |  |
| C | 663 N/mm |  |
| D | 450 N/mm |  |
| Q107 | A steel plate is 30 cm wide and 10 mm thick. If the diameter of the bolt hole is 20 mm, the net section area of the plate is: | 28 cm2 |
| A | 30 cm2 |  |
| B | 28 cm2 |  |
| C | 280 cm2 |  |
| D | 300 cm2 |  |
| Q108 | A closely spaced beams supporting floors or roofs of building but not supporting the other beams is termed as: | Joist |
| A | Girder |  |
| B | Girt |  |
| C | Joist |  |
| D | Purlin |  |
| Q109 | A welded plate girder consists of : Top flange : 290x22mm, Bottom flange 215x22mm, web 900x8mm.If the girder is subjected to a UDL of 28KN/m, over a span of 16m, calculate the compressive bending stress. | 137 MPa |
| A | 137 MPa |  |
| B | 237 MPa |  |
| C | 257 MPa |  |
| D | 287MPa |  |
| Q110 | A column of effective height 6m is subjected to an axial force of 500KN together with a bending moment of 20 KN-m. Calculate the least minimum cross sectional area of the column required. | 12346mm2 |
| A | 12346mm2 |  |
| B | 22346mm2 |  |
| C | 42346mm2 |  |
| D | 62346mm2 |  |
| Q111 | What is the hole diameter for a bolt of 25 mm diameter? | 30mm |
| A | 25mm |  |
| B | 26mm |  |
| C | 28mm |  |
| D | 30mm |  |
| Q112 | Two plates of 14 mm and 12 mm are joined by fillet weld, the maximum size of fillet weld may be: | 10mm |
| A | 16mm |  |
| B | 10mm |  |
| C | 24mm |  |
| D | 28mm |  |