NETWORK SECURITY

- Find the solution of x2≡ 3 mod 11
a) x ≡ -9 mod 11 and x≡ 9 mod 11
b) x ≡ 9 mod 11
c) No Solution
d) x ≡ 5 mod 11 and x ≡ 6 mod 11

Answer: d

- Find the set of quadratic residues in the set –
Z11* = { 1, 2, 3, 4, 5, 6, 7, 8, 9, 10}
a) QR set = {1, 2, 4, 5, 9} of Z11*
b) QR set = {1, 3, 6, 5, 9} of Z11*
c) QR set = {1, 3, 4, 9,10} of Z11*
d) QR set = {1, 3, 4, 5, 9} of Z11*

Answer: d

- In Zp* with (p-1) elements exactly:
(p – 1)/2 elements are QR and
(p – 1)/2 elements are QNR.
a) True
b) False

Answer: a

- Find the solution of x2≡ 16 mod 23
a) x = 6 and 17
b) x = 4 and 19
c) x = 11 and 12
d) x = 7 and 16

Answer: b

- Find the solution of x2≡3 mod 23
a) x≡±16 mod 23
b) x≡±13 mod 23
c) x≡±22 mod 23
d) x≡±7 mod 23

Answer: a

- Find the order of the group G = <Z12*, ×>?
a) 4
b) 5
c) 6
d) 2

Answer: a

- In the order of group G= <Z20*, x>, what is the order of element 17?
a) 16
b) 4
c) 11
d) 6

Answer: b

- The order of group G= <Z9, x> , primitive roots of the group are –
a) 8 , Primitive roots- 2,3
b) 6 , Primitive roots- 5
c) 6 , Primitive roots- 2,5
d) 6 , Primitive roots- 5,7

Answer: c

- Which among the following values: 17, 20, 38, and 50, does not have primitive roots in the group G = <Zn*, ×>?
a) 17
b) 20
c) 38
d) 50

Answer: b

- Find the primitive roots of G=<Z11*, x>?.
a) {2, 6, 8}
b) {2, 5, 8}
c) {3, 4, 7, 8}
d) {2, 6, 7, 8}

Answer: d

- Φ(37)=
a) 24
b) 22
c) 13
d) 36

Answer: d

- 7(3+j) mod 19 =
a) 7j mod 19
b) 1 mod 19
c) 73 + 7j mod 19
d) All of the mentioned are true

Answer: a

- 3201 mod 11 =
a) 3
b) 5
c) 6
d) 10

Answer: a

- Find a number x between 0 and 28 with x^85 congruent to 6 mod 29.
a) 22
b) 12
c) 6
d) 18

Answer: c

- Find a number x between 0 and 28 with x85 congruent to 6 mod 35.
a) 6
b) 32
c) 8
d) 28

Answer: a

- Find x for the CRT when x= 2 mod 3; x= 3 mod 5; x = 2 mod 7
a) 33
b) 22
c) 23
d) 31

Answer: c

- The inverse of 49 mod 37 is –
a) 31
b) 23
c) 22
d) 34

Answer: d

- Six teachers begin courses on Monday Tuesday Wednesday Thursday Friday and Saturday, respectively, and announce their intentions of lecturing at intervals of 2,3,4,1,6 and 5 days respectively. Sunday lectures are forbidden. When first will all the teachers feel compelled to omit a lecture? Use CRT.
a) 354
b) 371
c) 432
d) 213

Answer: b

- 17 x2 = 10 ( mod 29 )
a) x = 3, 22 (mod 29)
b) x = 7, 28 (mod 29)
c) x = 2, 27 (mod 29)
d) x = 4, 28 (mod 29)

Answer: c

- x7 = 17 (mod 29)
a) x = 8, 9, 12, 13, 15, 24, 28 (mod 29)
b) x = 8, 10, 12, 15, 18, 26, 27 (mod 29)
c) x = 8, 10, 12, 15, 17, 24, 27 (mod 29)
d) x = 8, 9, 13, 15, 17, 24, 28 (mod 29)

Answer: b

- How many primitive roots are there for 19?
a) 4
b) 5
c) 3
d) 6

Answer: d

- Which is the smallest unit amongst the following with reference to the ATM-
a) transmission path
b) virtual path
c) virtual circuit
d) all are of the same size

Answer: c

- IEEE 802.8 is/was –
a) IBM Token Bus
b) Integrated Services LAN
c) Wireless LAN and Mesh
d) Fiber Optic TAG

Answer: d

- Which of the following was the IBM Token bus?
a) IEEE 802.10
b) IEEE 802.11
c) IEEE 802.1
d) IEEE 802.4

Answer: d

- A device which is used to boost the signal between two cable segments or wireless access points is
a) Booster
b) Repeater
c) Switch
d) Router

Answer: c

- A device that provides a central connection point for cables is –
a) Switch
b) Hub
c) Gateway
d) Proxy Server

Answer: c

- A device that is used to connect a number of LANs is –
a) Router
b) Repeater
c) Bridge
d) Switch

Answer: a

- HTTP stands for ________
a) Hash Text Transfer Protocol
b) Hyper Text Transfer Protocol
c) Hash Transfer Text Protocol
d) none of the mentioned

Answer: b

- SMTP stands for ________
a) Service Message Transmission Permission
b) Secure Message Transfer Protocol
c) Simple Mail Transfer Protocol
d) Simple Message Transfer Protocol

Answer: c

- UDP stands for ________
a) User Datagram Protocol
b) Used Data Protocol
c) Unified Definition Protocol
d) Undefined Diagnostic Protocol

Answer: a

- ICMP stands for ________
a) Internal Control Message Protocol
b) Internet Cipher Mail Protocol
c) Internal Cipher Mail Protocol
d) Internet Control Message Protocol

Answer: d

- SONET stands for ________
a) Secure Offline Network
b) Synchronous Optical Network
c) Service Offline Network
d) Secure Optical Netwrok

Answer: b

- Which of the following does not lie in the Application Layer of the TCP/IP Model?
a) HTTP
b) SMTP
c) RTP
d) UDP

Answer: d

- Which of these is a connection oriented service?
a) X.25
b) Frame Relay
c) ATM
d) All of the mentioned

Answer: d

- Which one of the following is not a higher –layer SSL protocol?
a) Alert Protocol
b) Handshake Protocol
c) Alarm Protocol
d) Change Cipher Spec Protocol

Answer: c

- Which one of the following is not a session state parameter?
a) Master Secret
b) Cipher Spec
c) Peer Certificate
d) Server Write Key

Answer: d

- The full form of SSL is
a) Serial Session Layer
b) Secure Socket Layer
c) Session Secure Layer
d) Series Socket Layer

Answer: b

- Which protocol is used to convey SSL related alerts to the peer entity?
a) Alert Protocol
b) Handshake Protocol
c) Upper-Layer Protocol
d) Change Cipher Spec Protocol

Answer: a

- In TLS padding cann be upto a maximum of –
a) 79 bytes
b) 127 bytes
c) 255 bytes
d) none of the mentioned

Answer: c

- An HTTP connection uses port _________ whereas HTTPS uses port ____________ and invokes SSL.
a) 40; 80
b) 60; 620
c) 80; 443
d) 620; 80

Answer: c

- Which layer in the IEEE 802.11 protocol stack has the function of flow control and error control?
a) Physical Layer
b) Logic Link Control Layer
c) Medium Access Layer
d) None of the mentioned

Answer: b

- The correct order of the of the MAC header is-
a) MAC Control, Destination MAC Address, Source MAC Address
b) Destination MAC Address, Source MAC Address, MAC Control
c) Source MAC Address, Destination MAC Address, MAC Control
d) none of the mentioned

Answer: a

- Reliable data delivery and Wireless access control protocols are functions of which layer?
a) Physical Layer
b) Logic Link Control Layer
c) Medium Access Layer
d) None of the mentioned

Answer: c

- Which layer keeps track of the frames that have been transmitted and received?
a) Physical Layer
b) Logic Link Control Layer
c) Medium Access Layer
d) None of the mentioned

Answer: b

- CCMP uses which block cipher mode of operation?
a) OFB
b) CFB
c) CTR
d) CBC

Answer: c

- X.509 certificate recommends which cryptographic algorithm?
a) RSA
b) DES
c) AES
d) Rabin

Answer: a

- CMP stands for
a) cipher message protocol
b) cipher management protocol
c) certificate message protocol
d) none of the mentioned

Answer: d

- CMS stands for
a) cipher message syntax
b) certificate message session
c) cryptographic message syntax
d) none of the mentioned

Answer: c

- In terms of the size of the network the correct order (ascending) is –
a) PAN, MAN, LAN, WAN
b) LAN, MAN, WAN, PAN
c) PAN, LAN, MAN, WAN
d) LAN, PAN, MAN, WAN

Answer: c

- How many layers are there in the OSI reference model?
a) 4
b) 5
c) 6
d) 7

Answer: d