# Assignment 1

The due date for submitting this assignment has passed.

Due on 2018-08-15, 23:59 IST.

### Assignment submitted on 2018-08-15, 22:45 IST

2 points

A GSM900 cell tower antenna has 15 dBi gain and it is transmitting 20 W of power at 950 MHz. The power density at a distance of 50m in the direction of maximum radiation is:

100.6 mW/m2m2

20.1 mW/m2m2

9.6 mW/m2m2

3.6 mW/m2m2

### No, the answer is incorrect. 
Score: 0

### Accepted Answers:

20.1 mW/m2m2

2 points

Transmitting and receiving antennas operating at 5.8 GHz with gains of 18 dBi and 15 dBi, respectively, are separated by a distance of 2 km. If the transmitted power is 20W, then the received power is:

-67.7 dBm

-46.4 dBm

-37.7 dBm

-19.1 dBm

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

-37.7 dBm

2 points

The output power of low noise amplifiers is:

> 40 dBm

30 dBm to 40 dBm

10 dBm to 30 dBm

< 10 dBm

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

< 10 dBm

2 points

The carrier frequency of GPS receiver module is:

915 MHz

1575 MHz

1875 MHz

1915 MHz

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

1575 MHz

2 points

GPS module can be used in:

Vehicle Tracking

Remote Monitoring

Location Identification

All of these

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

All of these

2 points

The frequency range of AM radio is:

530 KHz – 1620 KHz

88 MHz – 108 MHz

180 MHz – 220 MHz

2.4 GHz – 2.5 GHz

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

530 KHz – 1620 KHz

2 points

In USA, the maximum allowed SAR limit for 6 minutes of usage is:

1 W/Kg

1.6 W/Kg

2 W/kg

2.4 W/Kg

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

1.6 W/Kg

2 points

The extremely high radiation zone of a cell tower is at a distance:

< 50 m

50 m to 300 m

300 m

None of these

### No, the answer is incorrect. 
Score: 0

### Accepted Answers:

< 50 m

2 points

In 2011, WHO has classified cell phone radiation as:

Class 1- carcinogen to human

Class 2A- probable carcinogen to human

Class 2B- possible carcinogen to human

Class 3- not classifiable as to its carcinogenicity to human

### Yes, the answer is correct. 
Score: 2

### Accepted Answers:

Class 2B- possible carcinogen to human

2 points

The Excessive use of cell phone over long period of time may cause:

Hearing problem

Increase in cancer risk

Irreversible infertility

All of these