Question Bank-2019

Optical Fiber communication

1. Multimode step index fiber has
a) Large core diameter & large numerical aperture
b) Large core diameter and small numerical aperture
c) Small core diameter and large numerical aperture
d) Small core diameter & small numerical aperture                          (a)

2) A typically structured glass multimode step index fiber shows as variation of attenuation in range of
a) 1.2 to 90 dB km-1at wavelength 0.69μm
b) 3.2 to 30 dB km-1at wavelength 0.59μm
c) 2.6 to 50 dB km-1at wavelength 0.85μm
d) 1.6 to 60 dB km-1at wavelength 0.90μm                                         (c )

3. A multimode step index fiber has a large core diameter of range
a) 100 to 300 μm
b) 100 to 300 nm
c) 200 to 500 μm
d) 200 to 500 nm       					               (a)

4.  Multimode graded index fibers are manufactured from materials with
a) Lower purity
b) Higher purity than multimode step index fibers.
c) No impurity 
d) Impurity as same as multimode step index fibers.                         (b)

5. Multimode graded index fibers have overall buffer jackets same as multimode step index fibers but have core diameters
a) Larger than multimode step index fibers.
b) Smaller than multimode step index fibers.
c) Same as that of multimode step index fibers.
d) Smaller than single mode step index fibers.  			(b)

6. Multimode graded index fibers use incoherent source only. State whether the following statement is true or false.
a) True
b) False                       						(b)

7. 10. In single mode fibers, the most beneficial index profile is
a) Step index
b) Graded index
c) Step and graded index
d) Coaxial cable							(b)

8.The fibers mostly not used nowadays for optical fiber communication system are
a) Single mode fibers
b) Multimode step fibers
c) Coaxial cables
d) Multimode graded index fibers					(d)

9. A fiber which is referred as non-dispersive shifted fiber is
a) Coaxial cables
b) Standard single mode fibers
c) Standard multimode fibers
d) Non zero dispersion shifted fibers				 	(b)

10. Standard single mode fibers (SSMF) are utilized mainly for operation in
a) C-band
b) L-band
c) O-band
d) C-band and L-band					                              (c )

11.Fiber mostly suited in single-wavelength transmission in O-band is
a) Low-water-peak non dispersion-shifted fibers
b) Standard single mode fibers
c) Low minimized fibers
d) Non-zero-dispersion-shifted fibers					(b)

12. Who proposed the idea of transmission of light via dielectric waveguide structure?
a) Christian Huygens
b) Karpon and Bockham
c) Hondros and debye
d) Albert Einstein      							(c )

8.Who proposed the use of clad waveguide structure?
a) Edward Appleton
b) Schriever
c) Kao and Hockham
d) James Maxwell							(c )

9. Which law gives the relationship between refractive index of the dielectric?
a) Law of reflection
b) Law of refraction (Snell’s Law).
c) Millman’s Law
d) Huygen’s Law							               (b)

10. The light sources used in fibre optics communication are :
a) LED’s and Lasers
b) Phototransistors
c) Xenon lights
d) Incandescent                                 					              (a)

11. The ________ ray passes through the axis of the fiber core.
a) Reflected
b) Refracted
c) Meridional
d) Shew								                (c )

12. Light incident on fibers of angles________the acceptance angle do not propagate into the fiber
a) Less than
b) Greater than
c) Equal to
d) Less than and equal to						              (b)

13. What is the numerical aperture of the fiber if the angle of acceptance is 16 degree
a) 0.50
b) 0.36
c) 0.20
d) 0.27									                (d)

14. The ratio of speed of light in air to the speed of light in another medium is called as_________
a) Speed factor
b) Dielectric constant
c) Reflection index
d) Refraction index							(d)

15. When a ray of light enters one medium from another medium, which quality will not change
a) Direction
b) Frequency
c) Speed
d) Wavelength								(b)

16. Dispersion – Chromatic Dispersion “.

1. What is dispersion in optical fiber communication?
a) Compression of light pulses
b) Broadening of transmitted light pulses along the channel
c) Overlapping of light pulses on compression
d) Absorption of light pulses 						(b)

17. What does ISI stands for in optical fiber communication?
a) Invisible size interference
b) Infrared size interference
c) Inter-symbol interference
d) Inter-shape interference 						(c)

18. For no overlapping of light pulses down on an optical fiber link, the digital bit rate BT must be:
a) Less than the reciprocal of broadened pulse duration
b) More than the reciprocal of broadened pulse duration
c) Same as that of than the reciprocal of broadened pulse duration
d) Negligible									(a)

19. The maximum bit rate that may be obtained on an optical fiber link is 1/3Γ. State whether the given statement is true or false.
a) True
b) False  								 (b)

20. 3dB optical bandwidth is always ___________ the 3dB electrical bandwidth.
a) Smaller than
b) Larger than
c) Negligible than
d) Equal to								(b)

21. A multimode graded index fiber exhibits a total pulse broadening of 0.15μsover a distance of 16 km. Estimate the maximum possible bandwidth, assuming no intersymbol interference.
a) 4.6 MHz
b) 3.9 MHz
c) 3.3 MHz
d) 4.2 MHz									(c)

22. What is pulse dispersion per unit length if for a graded index fiber, 0.1μs pulse broadening is seen over a distance of 13 km?
a) 6.12ns/km
b) 7.69ns/km
c) 10.29ns/km
d) 8.23ns/km						                         	(b)

23. Chromatic dispersion is also called as intermodal dispersion. State whether the given statement true or false.
a) True
b) False									(b)

24. The optical source used in a fiber is an injection laser with a relative spectral width σλ/λ of 0.0011 at a wavelength of 0.70μm. Estimate the RMS spectral width.
a) 1.2 nm
b) 1.3 nm
c) 0.77 nm
d) 0.98 nm									(c)

25. In waveguide dispersion, refractive index is independent of
a) Bit rate
b) Index difference
c) Velocity of medium
d) Wavelength									(d)

26.Non-Linear Effects”.

1. The nonlinear effects in optical fibers are large. State whether the given statement is true or false.
a) True
b) False 									(b)

27. How many categories of nonlinear effects are seen in optical fibers?
a) One
b) Two
c) Three
d) Four																							(b)

28.Which of the following is not related to Kerr effects?
a) Self-phase modulation
b) Cross-phase modulation
c) Four-wave mixing
d) Stimulated Raman Scattering 																			(d)

29. Linear scattering effects are _______ in nature.
a) Elastic
b) Non-Elastic
c) Mechanical
d) Electrical									(a)

30. Which thing is more dominant in making a fiber function as bidirectional optical amplifier?
a) Core material
b) Pump source
c) Cladding material
d) Diameter of fiber																					(b)

31. Which thing is more dominant in making a fiber function as bidirectional optical amplifier?
a) Core material
b) Pump source
c) Cladding material
d) Diameter of fiber								(b)

32. ________ semiconductor laser sources generally have broader bandwidths.
a) Injection
b) Pulsed
c) Solid-state
d) Silicon hybrid								(b)

33. Nonlinear effects which are defined by the intensity – dependent refractive index of the fiber are called as
a) Scattering effects
b) Kerr effects
c) Raman effects
d) Tomlinson effects								(b)

34. Self-phase modulation causes modifications to the pulse spectrum. State whether the given statement is true or false.
a) True
b) False									(a)

35. Self-phase modulation can be used for
a) Enhancing the core diameter
b) Wavelength shifting
c) Decreasing the attenuation
d) Reducing the losses in the fiber						(b)

36. The beating between light at different frequencies or wavelengths in multichannel fiber transmission causes
a) Attenuation
b) Amplitude modulation of channels
c) Phase modulation of channels
d) Loss in transmission								(c)

38. What is different in case of cross-phase modulation from self-phase modulation?
a) Overlapping but same pulses
b) Overlapping but distinguishable pulses
c) Non-overlapping and same pulses
d) Non-overlapping but distinguishable pulses                                                          (b)

39. When three wave components co-propagate at angular frequency w1, w2, w3, then a new wave is generated at frequency w4, which is given by
a) w4 = w1-w2-w3
b) w4 =w1+w2+w3
c) w4 =w1+w2-w3
d) w4 =w1-w2+w3									( c)

40. _____________ results from case of nonlinear dispersion compensation in which the nonlinear dispersion compensation in which the nonlinear dispersion compensation in which the nonlinear chirp caused by self-phase modulation balances, postpones, the temporal broadening induced by group velocity delay.
a) Four wave mixing
b) Phase modulation
c) Soliton propagation
d) Raman scattering									(d)